[image: image1.png]@

CYRUS

SYSTEMS
A Higher Standard




     CLEANING INSTRUCTIONS FOR CYRUS SYSTEMS ENTRANCE MATS

Your Systems can be cleaned using standard carpet cleaning methods, however it should be done frequently as the purpose of the system is to stop dirt from going into the facility.  A good cleaning program will not only extend the appearance of your system, but the general appearance of the facility as well.

The following steps are recommended to help you get the best performance from your Cyrus System.

1) Vacuum Daily  It is calculated that 79% of all dirt is particulate. Vacuuming will remove this dirt the most effectively.

2) Spot Removal  A program of commercial spot removal will ensure the continuing good appearance of the entrance system. The majority of spots can be effectively removed if they are attended to in the first 24 hours.
3) Hot Water Extraction  Using a pre-spray and then hot water extraction will remove any dirt that the vacuuming can not. This includes oils and greases that will cause the carpet fibers to mat together. Steam cleaning should be done periodically as the need arises.
4) Clean the Edges  The edge should be wiped down weekly using warm water. Sweep the dust and dirt from under the first couple inches around the outside edge of the tray.
5) Cleaning Under the Carpet System If you plan on storing your system for the summer, you may find on the floor small bits of stronghold residue. This is a  latex based product, and simply needs to be washed down with mild soap and warm water.
A maintenance plan that is put into place and followed is the most important part in ensuring the ongoing appearance of the Cyrus System.

Maintenance of Cyrus Entryway Carpets

Installation – Pick up - Storage
Maintenance: 

-commercial grade vacuum

1/day (*)

-hot water extractor 125lbs pressure
HOT WATER ONLY

                                                                 heavy traffic
:1/day

                                                                 medium
:every 2nd day

                                                                 light

:1/week

(*) In foul weather, the use of a “Shop Vac” may be necessary

-a damp cloth or mop on the edging and on the floor under the vinyl edge will remove any accumulation of rocks and debris that collects over time

-a regular maintenance program gives the best results
The use of rotary brushes is not recommended by the manufacturer of the carpet.  It could shorten the useful life of the carpet.  

A carpet is best kept in place 12 months/year, however, if stored, please note the following recommendations.

Installation:

-floor must be clean 

-unroll and slide all modules on a sheet of poly to their intended position.  Once all modules are in position

Note: because of the non-slip membrane, installing without a slipping liner (like poly) can be arduous.

Pick  up: 

-clean carpet and allow to dry, flip over and apply a sheet of poly on blue non-slip material and roll on tubes.  WHEN ROLLED, THE PILE WILL BE UP.  Failure to do this may result in damage to your mats.

Storage:

-horizontally

-clean, dry and roll pile up on 8” TUBES  in a dry place on a clean floor.  Do not stack

70% to 80% of dirt, dust and abrasives found in a building can be tracked to the entrances.  Entryway carpets stop and hold dirt and water and prevent them from spreading further in the building on the floor, on all stories, furnishings, etc.  A regular maintenance program of this one area protects/prolongs the finish on floors and lightens the maintenance duties and costs in the rest of the building.

(**) When the carpet is given to a sub-contractor for storage and/or cleaning outside your establishment, make sure these recommendations are well understood and applied.  Call Customer Service 800 661 3387 for technical support.  Cyrus cannot assume responsibility for handling by a third party and/or services rendered outside your establishment.

