1.0
Measuring for the Cyrus System

1.1
Exact Measurements

· Exact Measurements are needed for all Cyrus Systems

· Our product is dimensionally stable (i.e.: it wont stretch ¼ inch: Due to reinforced fibreglass and vinyl backing).

1.2
Tools Needed for Measuring

· Shipping Paper (2’ or 4’)

· Measuring Tape

· 1” Masking Tape

· Marking Pen

· 45 Deg. Square

1.3
Measuring: Basic

· Draw a basic diagram of the system noting any objects that may interfere with the system

· Measure the distances and add them to the drawing (measure within a ¼ inch)

· In most cases, some room should be left in-between a wall or object and our system (usually a couple of inches).   Most walls are not 100% straight and corners are usually not a perfect 90 deg.

1.4 Measuring: Advanced

· If the shape of the system is complex a Template may be needed

· A template is made from paper (General 2’ to 4’ shipping paper can be used) on the floor where the systems is to be placed.

· The paper is laid down on the floor and then objects or curves can be cut into the paper.   For large systems several pieces of paper may have to be taped together.

· If the system has to make contact with a wall or object, extra carpet should be indicated so it can be cut into place on sight.

2.0
Installation: No Modification Needed

2.1
Arrival of the System and Unpacking

· Systems arrive rolled around a tube covered in bubble wrap with a protective plastic sleeve

· First unwrap and unroll the system

· Let it sit for some time to let the carpet rest (usually 10 min.).

2.2 Moving the System in place

· To slide the system into its desired place, flap on end of mat to trap air under the system and then pull the system into place.  (For large systems several people will be needed due to the weight – air can be added by each person flapping at different positions around the system)

· Once the system is in place (must be exactly where it is needed as it will not stretch) roll up half the system and remove the white weave from under the system.   Some small mats do not have nylon weave.

· Repeat for the other half of the system.

· The friction from the stronghold will prevent the system from moving once it is walked on.

· The system can still be moved at this point by adding air under the system but it will be more difficult with the white weave removed.

3.0 Installation: System has to be Customized on Sight

3.1 Tools and Materials needed

· Heat Gun

· Carpet Knife

· Roller (Small metal)

· Tape Measure

· Cyrus “White” Glue

· Seaming Tape

· 45 Deg. Square

· Straight Edge (6’ / 12’)

3.2
Cutting the System

· Our system can be cut like any vinyl backed carpet

· As the system does not stretch exact cuts will need to be made.

3.3
Seaming
· Our system can be seamed together on sight

· Work from under the system (Tufting down)

· Remove 1.5” of Blue Stronghold( from edge of carpet if present. 

· Butt the 2 carpet pieces together

· Apply a thin layer of white glue to both edges of the carpet and let dry (3-4 minutes)

· Lay the black seaming tape over the seam – dull side down (white glue has been already applied to one side of this)

· Using a heat gun, heat the seaming tape then use a roller to apply weight to the joint.

· After the joint has cooled the seam is at full strength.

Note: As the friction and weight of our system holds the mat in place large pieces do not need to be seamed together.

3.4
Attaching nosing to the System

· Attaching the nosing involves the same principles as seaming the system together

· Apply 1.5” of glue to the edge of the vinyl backing (the nosing will come pre glued from the factory)

· Ensure the nosing is pushed against carpet before heat is applied (again using the roller).

3.5 Corners  - 45 Deg.

· Cut nosing at 45 deg. with square

· Apply a thin amount of glue to each inside edge of the angled cut on the nosing

· Heat the edges of the nosing and attach (hold for 15 seconds)

· Heat the corner down against the carpet

· Apply a thin layer of white glue on the underside of the corner (let dry)

· Apply seaming tape as per 3.3  (triangle pieces supplied).

4.0 Removal of the System

· Turn over the system (blue Stronghold( facing up / tufting down)

· Roll system on core tube

· Secure system with tape (ensure tape does not leave a residue)

· If any blue flakes are left on the floor after removal of the system wash affected area with soap and water.

